| Check maintenance records, check general condition - leaks, discolorization dents, chafing, nicks, corrosion or cracks. Verify Telatemps show no unexplained temperature increases during prior flight. |  |
| --- | --- |
| ## Cowl Door |  |
| Master switch | On |
| Oil pressure, alternator, governor lights, low fuel | On |
| Warning light test switches | Push to test |
| Fuel quantity | Check gages |
| Rotor brake | Actuation normal |
| Master switch | Off |
| Aux fuel tank quantity | Check |
| Fuel filler cap | Tight |
| Aux fuel tank | No leaks |
| Aux fuel drain | Sample |
| Gearbox oil | Full, no leaks |
| Gearbox Telatemp | Normal |
| Flex coupling | No cracks, nuts tight |
| Yoke flanges | No cracks |
| Sprag clutch | No leaks |
| Static source | Clear |
| Control rod ends | Free without looseness |
| Steel tube frame | No cracks |
| All fasteners | Tight |
| Tail rotor control | No interference |
| Cowl door | Latched |
| ## Engine Right Side |  |
| # Carb air ducts | Secure |
| # Carb heat scoop | Secure |
| # Engine sheet metal | No cracks |
| # Electrical terminals | Tight |
| # Fuel line | No leaks |
| # Oil cooler door | Check inside |
| # Oil lines | No leaks or chafing |
| # Exhaust system | No cracks |
| # Engine general condition | Check |
| # V-belt condition | Check |
| # V-belt slack | Check |
| # Sprag clutch | No leaks |
| # Upper bearing | No leaks |
| # Telatemp – upper bearing | Normal |
| # Lower sheave groove wear | Smooth & uniform |
| # Flex coupling | No cracks, nuts tight |
| # Yoke flanges | No cracks |
| # Steel tube frame | No cracks |
| # Tail rotor control | No interference |

| ## Engine - Rear |  |  |  |
| --- | --- | --- | --- |
| # Cooling fan nut | Pin in line w/ marks |  |  |
| # Cooling fan | No cracks |  |  |
| # Fan scroll | No cracks |  |  |
| # Telatemps – lower bearing | Normal |  |  |
| # Lower bearing | No leaks |  |  |
| Empennage |  |  |  |
| # Tail surfaces | No cracks |  |  |
| # Fasteners | Tight |  |  |
| # Position light | Check |  |  |
| ## Tail Rotor |  |  |  |
| # Gearbox Telatemp | Normal |  |  |
| # Gearbox | Oil full, no leaks |  |  |
| # Blades | Clean& no damage/cracks |  |  |
| # Rod ends | Free without looseness |  |  |
| # Pitch link jam nuts | Tight |  |  |
| # Teeter bearings | Check condition |  |  |
| # Teeter bearing bolt | Does not rotate |  |  |
| # Control bellcrank | Free without looseness |  |  |
| ## Tailcone |  |  |  |
| # Rivets | Tight |  |  |
| # Skins | No cracks or dents |  |  |
| # Strobe light condition | Check |  |  |
| # Antenna | Check |  |  |
| # Attachment bolts | Tight |  |  |
| ## Engine - Left Side |  |  |  |
| # Engine sheet metal | No cracks |  |  |
| # Exhaust system | No cracks |  |  |
| # Engine oil | 4-6 qts |  |  |
| # Fuel lines | No leaks |  |  |
| # Gascolator drain | Sample |  |  |
| # Throttle linkage | Operable |  |  |
| # Battery & relay | Secure |  |  |
| # Alternator belt tension | Check |  |  |
| # Steel tube frame | No cracks |  |  |
| # Engine general condition | Check |  |  |
| Fuel Tank (Main) |  |  |  |
| # Leakage | None |  |  |
| # Quantity | Check |  |  |
| # Filler cap | Tight |  |  |
| # Drain | Sample |  |  |
| ## Main Rotor |  |  |  |
| CAUTION – Do not pull rotor blades down as damage may occur. To lower one blade, push opposite blade up. |  |  |  |
| # Blades | Clean & no damage/cracks |  |  |
| # Pitch change boots | No leakage |  |  |
| # Main hinge bolts | Cotter pins installed |  |  |
| # All rod ends | Free without looseness |  |  |
| # Pitch link jam nuts | Tight |  |  |
| # Pitch link safety wire | Secure |  |  |
| # All fasteners | Tight |  |  |
| # Swashplate scissors | No excessive looseness |  |  |
| ## Turn rotors 90° and level |  |  |  |
| ## Fuselage Left Side |  |  |  |
| # Baggage compartment | Check |  |  |
| # Removable controls | Secure if installed |  |  |
| # Collective control | Clear |  |  |
| # Seat Belt | Check condition & fastened |  |  |
| # Door | Unlocked & latched |  |  |
| # Door hinge safety pin | Installed |  |  |
| # Landing gear | Check |  |  |
| # Ground handling wheel | Removed |  |  |
| # Position light | Check |  |  |
| ## Nose Section |  |  |  |
| # Pitot tube |  |  | Clear |
| # Windshield condition & cleanliness |  |  | Check |
| # Fresh air vent |  |  | Clear |
| # Landing lights |  |  | Check |
| ## Fuselage Right Side |  |  |  |
| # Landing gear |  |  | Check |
| # Ground handling wheel |  |  | Removed |
| # Position light |  |  | Check |
| # Door hinge safety pin |  |  | Installed |
| # Baggage compartment |  |  | Check |
| ## Cabin Interior |  |  |  |
| # Loose articles |  | Removed or stowed |  |
| # Seat belts |  | Check condition |  |
| # Instruments, switches, & controls |  | Check condition |  |
| CAUTION – Removable controls should be removed if person in left seat is not a rated helicopter pilot. |  |  |  |
| CAUTION – When flying solo, fill left baggage compartment to capacity before using right baggage compartment. Avoid placing objects in compartments that might injure occupants if seat collapses during hard landing.. |  |  |  |
| CAUTION – Shorter pilots may require cushion to obtain full travel of all controls. When using cushion, verify aft cyclic travel is not restricted. |  |  |  |
| ### Notes: STAND BACK AND LOOK AT ENTIRE AREA! |  |  |  |